| FET Change Request Form |  |  |
| --- | --- | --- |
| Information | Execution Date/Time | 2026/08/04 00:00 –06:00 |
|  | Objective | - 新增2 筆UE IP on Staging WAPGW |
|  | Detail Description | 配合EPG需求: 新增設備連到StagingMMS-WAPGW, 允許該設備UE IP連入MMS-WAPGW01S Impact node : MMS-DL360-7S |
|  | Change NE (Node) | MMS-WAPGW01S |
|  | Stage 驗證 (是否有Stage驗證, 若無-用取代機制.) | 請用下拉選單選擇 Stage 驗證機制 : Non live Pre -test 附註內容 : |
|  | SR Number |  |
|  | SR Requester | Chen, Vincent 陳翰宏 (514167) <vincentchen@fareastone.com.tw> |
|  | CR Level | c1 |
|  | C2以上執行時間例外 | L3同意02:00前執行 |

- Involved Party & RR Definition (Vendor 一定要有一個 人員為 CR Coordinator )

| FET or Vender | Vender Coordinator | Job Definition | Department Name | Name | Mobile Phone Number |
| --- | --- | --- | --- | --- | --- |
| FET | 請選擇 | CR executer & testing | NT-NO-VOO-MSO | Wayne Lu | 0955227256 |
| Taifon | Y | CR executer & testing | Engineer | David Xiao | 0973291018 |

- Implementation Procedure & Time Table(載明執行時間/內容與順序)

| Item | Start Time | End Time | Duration (min) | Action/Command | Responsible (廠商/FET + 人員名稱) | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 00:00 | 00:05 | 5 min | Pre Check Check ems Alarm https://10.76.129.118/cm/ check MMS-WAPGW01S MMS-DL360-7S 是否有 Alarm (做為CR前後Alarm比對) | FET-Wayne Taifon -David |  |
| 2 | 00:05 | 00:15 | 10 min | Pre Backup configuration 1登入MMS-WAPGW01S Switch user as root, and typing commands : 2執行備份 mkdir -p /home/apuser/temp/CR/CRxxxxxx/ cp -p /etc/ipset.conf /home/apuser/temp/CR/CRXXXXXX/ipset.conf.YYYYMMDD.before ip route show > /home/apuser/temp/CR/CRXXXXXX/route.YYYYMMDD.before cp /etc/sysconfig/network-scripts/route-eth0 /home/apuser/temp/CR/ CRXXXXXX /route-eth0.YYYYMMDD.before | FET-Wayne Taifon -David |  |
| 3 | 00:15 | 00:35 | 20 min | Implement - 登入 MMS-WAPGW01S Switch user as root, and typing commands : 2 Implement on MMS-WAPGW001S (新增二組IP 於ipset 之listppg) 2.1記錄上版前數量 ipset list listppg ¦wc -l 2.2新增ip於ipset中 ipset add listppg 10.196.253.128/27 ipset add listppg 10.196.253.160/27 2.3記錄上版後數量 ipset list listppg ¦wc -l 2.4檢查新增的IP 是否存在 ipset list listppg ¦grep 10.196.253.128/27 ipset list listppg ¦grep 10.196.253.160/27 2.5確認上述無誤後，執行 ipset Save ipset save > /etc/ipset.conf 3新增2組IP於 ip route(回給EPG設備) 3.1新增路由 vi /etc/sysconfig/network-scripts/route-eth0 10.196.253.128/27 via 10.76.128.215 dev eth0 10.196.253.160/27 via 10.76.128.215 dev eth0 3.2 Restart Network systemctl restart network 3.3檢查新增的IP 是否存在 ip route ¦grep 10.196.253.128/27 ip route ¦grep 10.196.253.160/27 | FET-Wayne Taifon -David |  |
| 4 | 00:35 | 00:40 | 5 min | Post Check N/A | FET-Wayne Taifon -David |  |
| 4 | 00:40 | 00:50 | 10 min | Post Backup configuration cp -p /etc/ipset.conf /home/apuser/temp/CR/CRXXXXXX/ipset.conf.YYYYMMDD.after ip route show > /home/apuser/temp/CR/CRXXXXXX/route.YYYYMMDD.after cp /etc/sysconfig/network-scripts/route-eth0 /home/apuser/temp/CR/ CRXXXXXX /route-eth0.YYYYMMDD.after | FET-Wayne Taifon -David |  |
| 5 | 00:50 | 00:55 | 5 min | Post Health check Check ems Alarm https://10.76.129.118/cm/ check MMS-WAPGW01S MMS-DL360-7S 是否有 Alarm (做為CR前後Alarm比對) | FET-Wayne Taifon -David |  |
| 6 | 00:55 | 01:00 | 5 min | KPI check N/A | FET-Wayne Taifon -David |  |
| 7 | 01:00 | 01:10 | 10 min | Regression test | FET-Wayne Taifon -David |  |

- Fallback Plan    (註明Fallback決定時間 & Fallback Criteria)

| Item | Start Time | End Time | Duration (min) | Fallback Action/Command | Responsible (廠商/FET + 人員名稱) | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 04:00 | 04:10 | 10 min | Fallback 1 Login server Login server by user account 2於 MMS-WAPGW01S / 刪除二組IP 於ipset 之listppg 2.1 登入MMS-WAPGW01S 2.2記錄退版前數量 ipset list listppg ¦wc -l 2.3刪除ip: ipset del listppg 10.196.253.128/27 ipset del listppg 10.196.253.160/27 2.4記錄退版後數量 ipset list listppg ¦wc -l 2.5檢查新增的IP 是否已不在 ipset list listppg ¦grep 10.196.253.128/27 ipset list listppg ¦grep 10.196.253.160/27 2.6確認上述無誤後，執行 ipset Save ipset save > /etc/ipset.conf 3刪除2組IP於 ip route(回給EPG設備) 3.1刪除路由(二組IP於 ip route) vi /etc/sysconfig/network-scripts/route-eth0 10.196.253.128/27 via 10.76.128.215 dev eth0 10.196.253.160/27 via 10.76.128.215 dev eth0 3.2 Restart Network systemctl restart network 3.3檢查新增的IP 是否已刪除 ip route ¦grep 10.196.253.128/27 ip route ¦grep 10.196.253.160/27 | FET-Wayne Taifon -David |  |
|  | 04:10 | 04:15 | 5 min | Post Check N/A |  |  |
| 2 | 04:15 | 04:20 | 5 min | Post Health check Check ems Alarm https://10.76.129.118/cm/ check MMS-WAPGW01S MMS-DL360-7S 是否有 Alarm (做為CR前後Alarm比對) | FET-Wayne Taifon -David |  |
| 3 | 04:20 | 04:25 | 5 min | KPI check NA | FET-Wayne Taifon -David |  |
| 4 | 04:25 | 04:30 | 10 min | Regression test | FET-Wayne Taifon -David |  |

- 監控機制 : System  Health check  /KPI Check/ regression call test (詳細 Test case)  ( 做為CR 前 與  CR 後 之 KPI & 服務功能不受影響 之確認)  - 請附  KPI 監控機制 ,  Health check and  Regression test 之 說明

System  Health check

KPI Check

regression call test

- Function SIT/UAT  (測試項目過多時可以附件夾檔於文件中)

- Capacity and Utilization Check before/after CR (e.g. route utilization from 60% to 90%)

- Service Impact Identify & Risk Analysis

- Plan B If CR executed must be success, please fill in alternative solution when CR failure

- Baby Sitting Monitoring Criteria

- 風險評估表

影響程度

| 高度(3) | 高度(6) | 極度(9) |
| --- | --- | --- |
| 中度(2) | 高度(4) | 高度(6) |
| 低度(1) | 中度(2) | 高度(3) |

非常嚴重(3)

嚴重(2)

輕微(1)

幾乎不可能(1)          可能(2)             幾乎確定(3)

發生機率

- Appendix Files